Edgeway Connector Pinout

| B/C |  |  |  | A |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| +5v |  | 33 |  | 1 |  | +5v |
| D15 | I/O | 34 |  | 2 | I/O | A23 |
| D14 | I/O | 35 |  | 3 | I/O | A22 |
| D13 | I/O | 36 |  | 4 | I/O | A21 |
| D12 | I/O | 37 |  | 5 | I/O | A20 |
| D11 | I/O | 38 |  | 6 | I/O | A19 |
| D10 | I/O | 39 |  | 7 | I/O | A18 |
| D9 | I/O | 40 |  | 8 | I/O | A17 |
| D8 | I/O | 41 |  | 9 | I/O | A16 |
| D7 | I/O | 42 |  | 10 | I/O | A15 |
| D6 | I/O | 43 |  | 11 | I/O | A14 |
| D5 | I/O | 44 |  | 12 | I/O | A13 |
| D4 | I/O | 45 |  | 13 | I/O | A12 |
| D3 | I/O | 46 |  | 14 | I/O | A11 |
| D2 | I/O | 47 |  | 15 | I/O | A10 |
| D1 | I/O | 48 |  | 16 | I/O | A9 |
| D0 | I/O | 49 |  | 17 | I/O | A8 |
| /BUSBUSY | I/O | 50 |  | 18 | I/O | A7 |
| /INSSTART | OUT | 51 |  | 19 | I/O | A6 |
| /ACK | IN | 52 |  | 20 | I/O | A5 |
| /IRQ | IN | 53 |  | 21 | I/O | A4 |
| /NMI | IN | 54 |  | 22 | I/O | A3 |
| /IRQPRI0 | IN | 55 |  | 23 | I/O | A2 |
| /IRQPRI1 | IN | 56 |  | 24 | I/O | A1 |
| /IRQPRI2 | IN | 57 |  | 25 | I/O | A0 |
| /BEL | OUT | 58 |  | 26 | OUT | /RD |
| /BEH | OUT | 59 |  | 27 | OUT | /WR |
| Reserved |  | 60 |  | 28 | OUT | /MREQ |
| Reserved |  | 61 |  | 29 | OUT | /IOREQ |
| /RESET | I/O | 62 |  | 30 | IN | /HALT |
| AUXV |  | 63 |  | 31 | OUT | /CPUHALT |
| 0v |  | 64 |  | 32 |  | 0v |

Control Signals

/BUSBUSY	Indicates that the backplane is free (HIGH) and not being used by any CPU (LOW)

A floating signal that is pulled high via a resistor.

Indicates whether the system bus is currently available. The method by which competing bus masters obtain ownership is defined separately by the bus arbitration protocol. This signal should not be used on its own as an indication that the bus is free and can be obtained.

/INSSTART	Indicates the start of an instruction read from memory.

/INSSTART is asserted by the CPU immediately before beginning execution of a new instruction and remains asserted until the first instruction-word memory access has commenced.

/ACK		Device acknowledgement of read, write or HALT operation

When a memory or I/O mapped device is accessed, the CPU will wait for the /ACK signal to be pulled LOW. Once the CPU sees this, it knows that the device has accepted the read/write operation and the CPU then deasserts the active /RD or /WR thus releasing the device. With /RD and /WR now high, the /ACK is released by the device. When /CPUHALT is asserted as a resumable internal halt and /HALT is not asserted, a HIGH-to-LOW followed by a LOW-to-HIGH /ACK handshake instructs the processor to resume execution. /ACK has no resume function for critical halt codes.

/RESET		Reset the entire system

Any device or any CPU can pull the /RESET signal low. This will cause all devices that monitor the /RESET signal and all /CPUs to restart.

/HALT		HALT the CPU

Pulling this signal low halts the CPU after the current instruction has executed and before the start of the next instruction. When the CPU has halted the /CPUHALT is pulled low by the CPU.

It’s possible to connect the /INSSTART signal to the trigger on a latch, and connect the latch output to the /HALT signal. This would cause the CPU to halt at the start of each instruction. If a “STEP” button is connected to the latch reset signal, the code being executed by the CPU can be single stepped.

/CPUHALT	The CPU has halted

This signal indicates that the CPU has halted execution. If the /HALT signal is also asserted when this signal is raised, it indicates that an external device has requested the CPU to temporarily halt. If the /HALT signal is not asserted however, this indicates that the CPU has internally detected a problem and has halted. The CPU must be RESET now to continue. In this second instance, the memory address associated with the fault is output onto A0 – A23, and the data bus contains an error code indicating the reason why the CPU halted.

Any data bus value of 255 or less is categorised by the CPU as a “resumable” halt. By pulling low the /ACK and then releasing it, the CPU will continue on from where it halted. These could be raised by the application software for debugging or other purposes. However, any halt code specified greater than 255 on the data bus is classed as a critical system halt and cannot be recovered from without a system RESET.

Notes:

Will add an assembler TRAP X instruction that can raise a 16 bit HALT event, however, when higher level languages are developed, they will only be able to generate resumable halts (0 to 255). If there is a need for external hardware to raise a critical fault, it can use one of the interrupt channels and a value on the data or address bus.

/IOREQ		Request I/O access

This signal from the CPU indicates that it’s wanting to access an I/O device at the address placed on the 24-bit address bus. Data can then be read/written to this device address via the /RD and /WR signals and the data bus.

For a write operation, the data to be written is loaded onto the databus

The CPU will publish the I/O address and then wait for the /ACK signal to be set by the device. Once set, the CPU will read the data from the databus if this is a read operation. In either case, the CPU will clear the /RD and /WR signals thus releasing the device.

/MREQ		Request memory access

This signal from the CPU indicates that it’s wanting to access memory at the address placed on the 24-bit address bus. Data can then be read/written to this memory address via the /RD and /WR signals and the data bus.

For a write operation, the data to be written is loaded onto the databus

The CPU will publish the memory address and then wait for the /ACK signal to be set by the device. Once set, the CPU will read the data from the databus if this is a read operation. In either case, the CPU will clear the /RD and /WR signals thus releasing the device.

Timings for accessing the backplane

Write To Memory

Check /BUSBUSY = HIGH

Check /ACK = HIGH

WRITE Address onto address bus

WRITE Data onto data bus

SET /MREQ = LOW

SET /WR = LOW

WAIT for /ACK = LOW

SET /WR = HIGH

SET /MREQ = HIGH

WAIT for /ACK = HIGH

End